- Energy Grants (Credits) Scheme Bill 2003

Energy Grants (Credits) Scheme (Consequential Amendments) Bill 2003

- Submission by Caltex Australia Limited

to the Senate Economics Legislation Committee

March 2003

Contact and authorising officer:

- Frank Topham

Government Affairs Manager

02 9250 5357

ftopham@caltex.com.au

Contents

|  | Page |
| --- | --- |
| - 1. Proposed amendments |  |
| 1.1 Background | 1 |
| 1.2 Details of proposed amendments | 1 |
| - 2. Benefits of cleaner petrol and diesel | 3 |
| - 3. Ultra low sulfur diesel |  |
| 3.1 Standards and technology | 4 |
| - 3.2 ULSD incentive | 5 |
| - 3.3 Form of ULSD incentive | 5 |
| - 4. Cleaner petrol and diesel after 2006 |  |
| - 4.1 10 ppm sulfur diesel | 6 |
| - 4.2 Early production incentive for 10 ppm sulfur diesel | 7 |
| 4.3 50 ppm sulfur petrol | 7 |
| - 4.4 Early production incentive for 50 ppm sulfur petrol | 7 |
| - 5. Details of operation of production subsidy | 8 |

- 1.  Proposed amendments

1.1  Background

Under the 1999 Measures for a Better Environment package, the Government noted that the Energy Grants Credit Scheme would provide encouragement for conversion to cleaner fuels.  The Treasurer has stated the Government is committed to pursuing options to achieve this and is doing so by examining the issue as part of the consideration of alternative fuels within the Energy Task Force.

Caltex welcomes the Government’s commitment.  However, there are relevant issues that can be addressed now by amendment of the bill, either to enable implementation of existing policy or to provide a framework for future decisions.  In particular, there is a need to provide incentives for cleaner petrol and diesel, in addition to alternative an renewable fuels.  The bill should therefore be amended to provide for:

- implementation of existing policy for an incentive for early introduction of ultra low sulfur diesel from 1 July 2003

- a framework for incentives for early introduction of cleaner petrol and diesel from 1 January 2006

- administration of the ethanol subsidy within the common legislative framework for grants schemes

- extension of grants to biodiesel

- extension of an existing policy position on use of petroleum products in road-making to products other than diesel.

1.2  Details of proposed amendments

Caltex proposes the Energy Grants (Credits) Scheme Bill 2003 (EGCS bill) be amended:

- Insert a Part 4A – Entitlement to cleaner fuel credits.  This part would define the entitlement to a production subsidy for ultra low sulfur diesel (ULSD), consistent with the incentives for early production of ULSD agreed as part of the Measures for a Better Environment.  Details of operation of a subsidy are discussed in Section 5.

- As part of the new Part 4A, provide a framework for entitlements to production subsidies for the early production of cleaner petrol and diesel from 2006 until the date of effect of mandated standards for that petrol and diesel (which are currently being determined).  The benefit of such enabling legislation is that once a government policy decision is taken, the decision can be put into effect quickly through regulation.

- Insert a Part 4B – Entitlement to biofuels credits.  This part would define the entitlement to a production subsidy for ethanol produced from biomass (such as sugar cane products, wheat products and wood), bringing the existing ethanol production subsidy within the consistent regulatory framework of the EGCS.

- As part of the new Part 4B, provide a framework for entitlement to production subsidies for biodiesel, including biodiesel blended with crude oil-derived diesel, provided the biodiesel and biodiesel/diesel blends meet national fuel quality standards (yet to be determined).

- Make consequential amendments to Part 5 so the amount of an energy grant (in the form of a production subsidy) can be calculated.  Part 5 would also enable amounts and durations of subsidies to be determined by regulation.  For example, consistent with existing policy, the ULSD subsidy would be 1 cent per litre (cpl) in calendar 2003 and 2 cpl in calendar years 2004 and 2005.  Cleaner petrol and diesel subsidies would be regulated once the government has made the relevant decisions on standards and subsidy amounts.

- A Section 53A should be added to allow for an off-road credit in respect of petroleum products other than diesel used in road-making.  Section 53(1) restricts the off-road credit to off-road diesel fuel and thereby precludes a credit being given in respect of other products such as heating oil and kerosene used in road-making. A credit for use of diesel in road making is provided by Section 53(6), so that use of heating oil and kerosene for such a purpose would be consistent with this section.

- 2.  Benefits of cleaner petrol and diesel

There is ample evidence of the adverse effects of motor vehicle emissions on human health.  Air quality is also a major public environmental concern.

For this reason, governments have progressively tightened regulation of vehicle emissions and fuels.  In Australia, the emissions of most concern are hydrocarbons (HC) and oxides of nitrogen (NOx), which in combination create photochemical smog, and fine particulates (PM).  Emissions of these pollutants are affected by the sulfur content of petrol.  Air toxics such as benzene, 1,3-butadiene and aldehydes are also of concern.

Emissions of these pollutants have been reduced by changes to engines and addition of anti-pollution devices, particularly catalytic converters, and will be reduced further by more complex anti-pollution technologies.  Changes to fuels have been necessary to enable vehicle technologies and to reduce emissions directly.

A familiar enabling change to fuel standards was the mandating of unleaded petrol for Australian cars from 1986 onwards, to complement the mandatory fitting of catalytic converters.

In the future, the most important technology-enabling fuel standard changes will be reduction of the levels of sulfur in both petrol and diesel.  This will allow for fitting of advanced anti-pollution devices to reduce NOx and PM emissions.  The other technology-enabling change (although not a fuel standard) will be greater use of higher octane petrol to allow for more fuel-efficient engines.

All particulate emissions are of concern but the focus of regulation is on PM emissions from diesel engines.  The level of PM emissions is somewhat related to the sulfur content of diesel.

In petrol engines, reduction of sulfur content reduces emissions of HC and NOx for a given engine technology.  In addition, sulfur content has an adverse effect on the efficiency of catalysts, so that emissions from older cars increase over time, even if they are well maintained.

Lower sulfur allows for design of more fuel-efficient engines, by allowing “lean burn” operation.  Lean burn engines generate high levels of NOx, which requires advanced anti-pollution equipment, which can only operate on very low levels of sulfur in petrol and diesel.  Greater fuel efficiency means reduction on the substantial greenhouse gas emissions from road transport.

In the future, cleaner petrol and diesel will achieve a number of objectives:

- reduce air pollution from the existing petrol and diesel vehicle fleets

- reduce air pollution and improved fuel efficiency (lower greenhouse gas emissions) from new vehicles

- enable the manufacture of cleaner, more fuel-efficient cars in Australia, because appropriate petrol will be available to fuel them

- allow for the import of cleaner petrol and diesel vehicles with advanced technologies.

If the fuel policy settings proposed in this submission were implemented, cleaner petrol and diesel could help underpin Australian oil refining by providing a competitive advantage over refiners in Asia that were not equipped to manufacture cleaner fuels.  By the use of early production incentives, this could be achieved without the risk of large additional costs to Australian motorists, as explained later in this submission.

A strong, competitive refining industry would increase Australia’s security of supply by ensuring cleaner fuels to meet local specifications were readily available at reasonable prices and help ensure the success of strategies for reduced air pollution and greenhouse gas emissions from vehicles.

- 3.  Ultra low sulfur diesel

- 3.1  Standards and technology

Ultra low sulfur diesel (ULSD) refers to diesel with a sulfur content not exceeding 50 parts per million (ppm) by mass.  ULSD has been mandated in Australia from 1 January 2006 under the Fuel Quality Standards Act.  The current standard for diesel, which came into effect on 1 January 2003, sets a maximum of 500 ppm sulfur in diesel.  These diesel sulfur levels are the same as the 2005 European standard (“Euro 4”) and 1996 European standard (“Euro 2”) respectively.

The timing of the fuel standards is aligned with tighter Australian emission standards for diesel vehicles.  However, as explained above, there is significant overall benefit from reduction of sulfur level in diesel used by the existing vehicle technologies, not just new vehicle technologies.  New technologies achieve greater emission reduction per vehicle from lower sulfur but there are few vehicles initially – it takes many years for the fleet to be renewed.  The existing fleet achieves a smaller reduction in emissions per vehicle but the reductions are achieved by the whole fleet, so there is a significant total reduction in emissions.

The date from which ULSD is mandated – 1 January 2006 – reflects vehicle technology requirements and refinery supply capabilities.  Euro 4 vehicles are mandated in Europe from 2005 and equivalent Australian diesel vehicles (ADR 80/01) from 2006/7.  Australian refineries require major investments to produce ULSD, with a typical 3 year lead time from initial planning to production.

However, given the benefits from reduction of existing fleet emissions and the potential for production and import of vehicles ahead of the European standards, it makes sense to encourage the earlier production of ULSD from Australian refineries.

- 3.2  ULSD incentive

Measures for a Better Environment (MBE) provides for an incentive for the production (or import) of ULSD before 2006, in the form of an excise differential.  MBE proposed that excise on regular diesel (maximum 500 ppm sulfur) be increased by 1 cpl in calendar 2003 and 2 cpl in 2004 and 2005.  If ULSD and regular diesel sell at the same price, as seems likely (because consumers will generally not pay a premium for improved environmental quality in fuel), the effect of the lower ULSD excise will be to allow refiners to recover the increased production cost of ULSD.  The effect will be to encourage refiners to invest in ULSD ahead of the mandated date.

On 24 December, the Treasurer announced that implementation of the ULSD production incentive would be deferred for 6 months, until 1 July 2003.  Accordingly, there would be no increase in diesel excise.  The reason given was the impact of increased diesel costs on farmers suffering from the drought.  Caltex’s view is the ULSD incentive could have been implemented, as increased excise collection in 2003 would have allowed for a temporary increase in the Diesel Fuel Rebate for farmers, offsetting the diesel price increase.  Such short-term compensation for farmers could also be implemented with the ULSD incentive from 1 July 2003, without net impact on government revenues, if believed necessary.

The effect of the Treasurer’s announcement, despite repeated prior assurances that the ULSD incentive would be implemented on time, has been to undermine Caltex’s confidence that the government intends to implement the ULSD incentive at any time.  If the ULSD incentive is guaranteed by an amendment to the EGCS bill, this will provide sufficient certainty for Caltex to implement plans for production of some ULSD from mid 2003 and consider investments to increase production as soon as possible.  Full ULSD production will occur closer to 2006 as part of investments to meet the 2006 national fuel standards.

Other refiners are on the record as have already invested for ULSD or having plant under construction.  However, because of the additional operating costs associated with ULSD production, it seems unlikely they will produce any ULSD without an incentive.  In addition, deferral of the ULSD incentive must reduce the confidence of all refiners when considering any future investment that is dependent on tax incentives.

Caltex’s estimate, based on publicly available information plus knowledge of our own capabilities, is that ULSD production with an incentive in place would be 50% of all diesel in the financial year 2003-04, 80% in 2004-05 and 100% in 2005-06.  The incentive would therefore be very effective in accelerating production of ULSD.

Given the deferral of the ULSD incentive and effect on industry confidence, legislation of the incentive with the EGCS bill would give the necessary confidence for Caltex to plan for some ULSD production from 1 July 2003 and firm up plans to increase ULSD production well ahead of the 2006 mandate.

- 3.3  Form of ULSD incentive

The ULSD incentive was envisaged in MBE as an excise differential.  However, there are good reasons for implementing the incentive as a production subsidy, as discussed in the 2002 report of the government’s Fuel tax inquiry.  The incentive effects of a production subsidy are equivalent to an excise differential and there are no differences in the effect on consumer prices.

As a matter of general taxation policy, it would be desirable for substitutable fuels to be subject to excise and subsidies paid to fully or partially offset the excise as the government believes appropriate.  This type of scheme applies to eligible on- and off-road use of diesel, other fuels under the Diesel and Alternative Fuels Grants Scheme, petrol and diesel eligible for the Fuel Sales Grant and ethanol.  Caltex proposes it be extended to early production of ULSD and future cleaner petrol and diesel, and biodiesel.

Caltex does not support inclusion of LPG under such arrangements as LPG can not be used in unmodified engines or blended with petrol and diesel.  Moreover, Caltex does not support any effective excise being imposed on LPG.  From an administrative point of view it is simpler to leave it outside the excise system.

Section 5 discusses how a production subsidy would work in practice.  Conceptually, it is the same as the schemes contained in the EGCS bill, in that full excise is paid and a grant is obtained by eligible recipients.  The details are different, in that the recipient of the grants is a producer or importer, rather than an end-user, and the amount of the grant is calculated on production or imports, rather than fuel purchases.  However, the eligibility framework of the EGCS bill and the administrative  provisions of the  Product Grants and Benefits Administration Act 2000 (PGBAA) are well suited to provision of production incentives for cleaner petrol and diesel.

- 4.  Cleaner petrol and diesel after 2006

4.1  10 ppm sulfur diesel

The government is currently assessing the vehicle emission and fuel standards that should apply after the currently regulated standards, through the Motor Vehicle Environment Committee (MVEC).

Australia is committed to harmonisation with European emissions standards, although there is no agreed period over which this should occur.  It seems likely that Australia will adopt diesel truck standards that take into account the advanced standards that will apply in Europe, the United States and Japan later in the second half of the decade, as all of Australia’s diesel truck engines are imported.

These trucks will require diesel with a maximum 10 ppm sulfur.  The latest timing for supply of diesel will depend on the timing of emissions regulations.  Caltex therefore anticipates a diesel standard with maximum 10 ppm sulfur around the end of the decade.  10 ppm sulfur diesel is mandated in Europe from 2009, with phase-in from 2005.

We support a 10 ppm sulfur standard as we believe the strategic direction of the Australian refining industry must be towards cleaner fuels for competitive reasons, as well as the benefits to air quality and greenhouse gas emissions.

It may be possible for the standard to be advanced somewhat before the end of the decade, however there must be an appropriate gap between the 2006 ULSD standard and the 10 ppm sulfur standard for diesel.  If the standard were advanced too far in time, it would be an example of sovereign risk, by effectively overriding the earlier ULSD standard on which refiners have based their investments.  It would also risk price spikes due to lack of availability of imports to the tighter 10 ppm standard, as Australian product prices are determined relative to import competition.

- 4.2  Early production incentive for 10 ppm sulfur diesel

A production incentive from 2006 would encourage production of 10 ppm sulfur  diesel ahead of the mandated date, while avoiding the risk of price spikes.  This is because consumers in general are not prepared to pay more for cleaner fuels, so would not pay more for 10 ppm sulfur diesel than regular (at that time) 50 ppm sulfur diesel (ULSD).  However, refiners would have an incentive to supply 10 ppm sulfur diesel if a production subsidy at least covered their costs (allowing for market risks and a reasonable profit margin) and their financial circumstances allowed the investment to take place.

The production subsidy for 10 ppm sulfur diesel would operate in exactly the same way as the ULSD subsidy, with a smooth transition from one to the other on 1 January 2006.  The amount of the 10 ppm sulfur diesel subsidy would need to be determined but would not exceed the 2 cpl ULSD subsidy and could be significantly less.  The subsidy would terminate once 10 ppm sulfur diesel was mandated.

4.3  50 ppm sulfur petrol

The sulfur standard for petrol will be tightened to a maximum 150 ppm by weight from 1 January 2005.  It seems likely that emission standards for petrol-engined vehicles will be tightened later in the decade, requiring petrol with a maximum 50 ppm sulfur.  This is the sulfur standard that will apply in Europe from 2005.

Maximum 10 ppm sulfur petrol is mandated in Europe from 2009, with a phase-in from 2005, but it is far from clear that Australia should adopt vehicle emission standards requiring such petrol in any similar timeframe.

- 4.4  Early production incentive for 50 ppm sulfur petrol

Caltex proposes a production subsidy be provided for 50 ppm sulfur petrol (ULSP), analogous to the subsidy for 10 ppm sulfur diesel.  The reduced sulfur level would benefit the existing fleet by reducing emissions and extending effective catalyst life, as well as enabling new technologies.  The availability of  ULSP would also encourage the import of advanced, low pollution, fuel efficient vehicles that otherwise could not be imported because of lack of appropriate fuel.

The amount of the subsidy would need to be determined.  In general, reduction of sulfur in petrol in significantly more expensive than for diesel because reduction of petrol sulfur adversely affects other petrol characteristics (especially octane).  However, a figure of 2 cpl could be taken as indicative at this time, recognising the need for further analysis.

- 5.  Details of operation of production subsidy

Caltex has proposed the ULSD incentive be implemented as a production subsidy and that future production subsidies be provided for the early production of cleaner petrol and diesel from 2006.  The following discussion shows how the ULSD production subsidy would work in practice and illustrates how similar subsidies could operate.

A  production subsidy for ULSD is the most effective and administratively straight forward way of achieving the government commitment in the MBE to bring forward supply of ULSD to the Australian market:

- the subsidy to suppliers would be 1 cpl in 2003 and 2 cpl through 2004 and 2005

- the excise increase required to finance the subsidy would be estimated at the beginning of each calendar by the ATO in consultation with suppliers

- any under- or over-estimates of excise in a particular year would be taken into account in the excise calculation for the following year.

- Caltex estimates the excise increase would be 0.8 cpl in 2003-04, 1.6 cpl in 2004 in 2004-05 and 2.0 cpl in first half 2005 (the subsidy for ULSD would terminate on 31 December 2005).

Eligible production of ULSD would be measured at some point between refinery production and distribution terminal.  This would be on the basis of self-assessment acceptable to the ATO and capable of being audited by the ATO, as provided for in the PGBAA.  Key requirements would be appropriate metering and quality testing (for sulfur and other properties to the current diesel standard).

The subsidy would be paid only on the volume of diesel containing less than 50 ppm sulfur of merchantable quality i.e. complying with the national fuel quality standard applicable at the time of sale.  Details of metering and sampling should not be prescribed in regulation but would need to be acceptable to the ATO as a basis for self-assessment.

The subsidy would be paid to the entity producing (or importing) the ULSD.  Under an excise differential option, the benefit would accrue to marketer ex-terminal (the point at which excise is paid), so that the link between production and the production incentive would be indirect.

Under a production subsidy, excise would continue to be measured at the loading rack, as there would be no link between the subsidy and excise, except through the Commonwealth Government budget.  All excise arrangements would be unaffected.

The government does not intend that ULSD be required to be sold separately from regular diesel, as this could be impractical (requiring two grades of diesel in some cases, for which there is insufficient service station tankage) and unnecessary from an environmental viewpoint. ULSD could still be made available as a separate product but this would be driven by market competition.

There is no difference in the environmental benefit between a given quantity of straight ULSD and the same quantity of ULSD blended into regular diesel.  This is because the reduction in particulate emissions is directly proportional to the level of sulfur in diesel with current technology engines.  This situation will change once Euro 4 diesel trucks are available from 2006/7 but all diesel will be ULSD by regulation from 2006.  Accordingly, there is no reason why the same subsidy should not apply to straight or blended ULSD.

Treasurer’s media release 13 February 2003.  The same statement is in the Second Reading speech.

It could be argued that as the ULSD incentive was intended to take effect from 1 January 2003 but has been deferred until 1 July 2003, the value of the incentive in calendar 2003 should be maintained by increasing the rate in the second half of 2003 to 2 cpl.

Caltex can direct the committee to sources of information on this issue, if required.

The total investment by Australian refineries for the package of tighter fuel standards through 2006, including ULSD and reduced benzene in petrol, exceeds $1 billion